SASMEE Locomotive Efficiency Trials Day 2003

By

Allan Wallace

14/6/2003

Bob Smith and SAR 702 start their dynamometer run

CONTENTS

Introduction	2

The Dynamometer Car	2

How the trials were conducted	3

Results	4

Comments	5

Notes on the winning locomotive.	6

Acknowledgements	7

# Introduction

Some years ago I was approached by fellow model engineer John Lyas about jointly building a model of the SAR/VR dynamometer car which is currently preserved at the Port Dock Railway Museum in Adelaide, South Australia.  In due course, the model was completed and it has been seen at several conventions and tracks.  I gave a talk at a club meeting recently and several members expressed an interest in holding a locomotive efficiency trials day, along the lines of an annual event held in the UK and known as “The International Model Locomotive Efficiency Competition” (IMLEC).

A date was fixed, a roster drawn up, officials appointed and a manual created.  Saturday 14 June 2003 became the first, (but not the last!), SASMEE Locomotive Efficiency Trials Day.

# The Dynamometer Car

Externally, it is a close 5” gauge replica of the prototype in its early livery.  Internally, it is very modern.  The front drawbar is linked to an electronic load cell, and the wheels drive a DC tachometer and an optical revolution counter.  A box of electronics conditions the signals, converts them to digital format, and transmits them to a laptop computer carried on a conventional passenger truck behind the car.

The laptop computer displays a scrolling “paper” chart, like the original instrument table, but it reads data every second and stores the information on disk for subsequent display or printing.

A full technical description of the dynamometer car can be downloaded from my website, at www.avocetconsulting.com.au/modeleng.

Figure 1 is a typical display.

Figure 1

# How the trials were conducted

The test track was the outer loop at SASMEE, catering for 5” and 7.25” gauge.  Locomotives towed the dynamometer car and a train for about 30 minutes. Coal was taken from a weighed batch and the firebox refilled to its starting depth at the end of the run.  Drivers could choose their rolling stock and load, and were free to change it through the run.

Only the load from the dynamometer car back was included in the work done tally.  Two of the 5”g engines were not set up for tender riding, so those drivers sat on the dynamometer car and hence were included in the train load (to their advantage).

The grades around the circuit are not severe, but generally the drawbar pull becomes a push at some places when the locomotive slows down and the train pushes the engine.  The software was set to ignore negative drawbar force so that locomotive braking did not reduce the work tally.

To calculate the thermal efficiency, the useful work done at the drawbar (measured by the dynamometer car, in Joules) was divided by the energy released by the burnt coal.  This was obtained from the mass of coal used, multiplied by the adopted calorific value of 30 MJ/kg.  The fuel used for all engines is actually a processed coal known here as “char”.

# Results

A complete set of individual results for each engine can be downloaded from www.avocetconsulting.com.au/modeleng.   Figure 2 shows the relative thermal efficiencies in rank order.  These figures compare well with those published from other trials, including the IMLEC.

Figure 2

Efficiency is the ratio of output/input, so it is a valid index for comparing locomotives of any scale.  Figure 3 breaks the efficiency into its two components for each engine, where drawbar power is the Output, and coal rate is a measure of Input.

Figure 3

If the engines had been loaded equivalent to their potential performance levels, one would expect the larger locomotives to use more fuel, and do more work, than the smaller ones.

In figure 4  the maximum force, power and speed values are the peaks measured over a one-second period, and can be subject to distortion due to train dynamics.

Figure 4

In figure 5 the coal burnt, and the work done, have both been divided by the running time to express them as watts, and kg/hr, which allow direct comparison.   The thermal efficiency can be obtained by dividing the average drawbar power by the coal rate (after conversion to watts).  The last column signals whether the driver’s mass was part of the train or not.

Figure 5

# Comments

This is the first trails day of its kind at SASMEE and it has been most instructive.  Clearly, most of the engines were not working very hard, which would adversely affect their thermal efficiency.  There would be an optimum power output where the efficiency is a maximum for any engine.  One of the challenges for drivers is to choose a load near to this optimum.  The mass of the train is not the only determinant of how hard the engine must work, since flange friction and dragging train brakes can contribute or even dominate the drawbar pull required.  Inspection of the individual datalogs showed that Speedy was working against moderately dragging train brakes, and even had to pull the train downhill!  For this reason this run produced the highest work done.

One issue that is not resolved is how to account for the driver’s mass.   When the driver sits on the tender, ahead of the dynamometer drawbar, the work done by the engine to carry the driver is not recorded.  This becomes a significant penalty for small locos and heavy drivers. There’s an opportunity for jockeys here!    On the other hand, Speedy and the Gresley had no provision for tender riding and were driven, perhaps inelegantly, from the roof of the dynamometer car, which is fitted with pedals and a foot brake.  There is a suggestion to make a strain-gauged link that can be placed ahead of the driver, but it is most unlikely to be universal for every locomotive.

We felt that the biggest shortcoming in the procedure was the measurement of coal consumed.  Even with the best intentions, one cannot be sure that the level of fuel in the firebox is the same before and after the run.  The accuracy can only be improved by running longer or harder, and thereby burning more coal. Ideally, the coal consumed should be several times the volume of the fire.  Koppel did not require any firing for the entire duration, because the mass of its fire was large compared to the coal consumed.  This is a sure sign that Koppel’s train was far too light to give it a proper run.  At the other extreme, Speedy was working quite hard, and burnt about twice the mass of coal in her fire.

Drivers were directed to start with a full firebox, then refill with weighed coal to the same level at the end of the run.  In most cases, the engines steamed so hard under the steady conditions that it was impossible to prevent significant steam loss from the safety valves.  In these engines it would have been better to start and end with a lower level of fire. Of course, it is even more difficult to judge a partial refill!

Driving technique is a significant factor in the results and we all noted how our engines were different to drive under sustained non-stop conditions.  We have developed our techniques and our engines to perform on field days, rather than for efficiency trials.  In these unfamiliar circumstances I am sure we do not see the true potential of the engines, and some experience and practise will increase the measured efficiencies.

When one watches a video of the IMLEC event, it seems that drivers are intent on running the maximum possible load at the maximum possible speed.  This certainly maximises the work done, but it should be remembered that the operating conditions for maximum power are not necessarily the same as the conditions for maximum efficiency.

# Notes on the winning locomotive.

I always intended to briefly describe the winning locomotive, and feel mildly self-conscious that it was mine.  My fellow model engineers have been most polite about it (thank you) and I trust that this section won’t be seen as indulgence on my part!

Speedy was designed by LBSC and took 600 hours over a period of 3 years to build.  The valve gear and motion details were closely based on K.N.Harris’s redesign.

There are needle roller bearings in the axleboxes, running on case-hardened axles.  The cylinders (1.625” bore x 2.125” stroke) are bronze with plain stainless steel piston valve bobbins, without rings, according to LBSC’s practice.  The pistons have two adjacent packing grooves, one with graphited yarn and the other with a Viton O ring.

The boiler is copper, with a screw-type regulator.  Superheat is very high, with five stainless steel elements projecting right through the firebox for maximum radiant heat input.  The grate is in cast iron and has a 50/50 gap/bar ratio at ¼” pitch.

I sleeved down the axle pump to a displacement ratio of 1:256 with the cylinder displacement. This is less than half the designed displacement but it still has more than ample capacity.  There is an injector fitted, which I rarely use.

There is a turbo generator fitted under the footplate, with a toroidal (zero magnetic cogging) armature.

According to the dynamometer car, her maximum output at the drawbar exceeds 500 watts, but at an average power of 250 watts she can be notched up for more efficient expansive running.

Figure 6  Speedy at the 2003 convention witb Allan Wallace and son Robin.

# Acknowledgements

Special thanks to the SASMEE committee, the helpers and the drivers.