# SASMEE Locomotive Efficiency Trials Day 2003

14/6/2003

Raw Dynamometer results

Allan Wallace

SASMEE Locomotive Efficiency Trials Day 2003	1

16:39:10 14/6/2003 Allan Wallace 5" Speedy 0-6-0T	2

15:11:47 14/6/2003 Barry Dunstan 7.25" Dolgoch 0-4-0	3

12:14:27 14/6/2003 Bill Coles 7.25" Jens 0-4-0	4

10:47:11 14/6/2003 Bob Nash 7.25" Koppel 0-6-0 Driven Glen	5

12:58:47 14/6/2003 Bob Smith 5" SAR 702 2-8-2	6

15:55:57 14/6/2003 Bryan Homann 5" Gresley 2-8-0	7

11:29:30 14/6/2003 Graham Driscoll 7.25" 4 truck Shay	8

14:29:50 14/6/2003 Ian Thomas 7.25" Freelance 4-6-2	9

13:46:6 14/6/2003 John Beal 5" Schenectady  0-6-0T	10

Important note: Due to a mistake in entering the dynamometer car wheel diameter, the figures for distance, speed, power and energy must all be reduced by 15%.

The results here are taken from the dynamometer car record only.  Comparative performance figures, including coal consumption and overall thermal efficiency, are published in a separate report.

# 16:39:10 14/6/2003 Allan Wallace 5" Speedy 0-6-0T

Important note: Due to a mistake in entering the dynamometer car wheel diameter, the figures for distance, speed, power and energy must all be reduced by 15%.

Summary values:

Max       Min

Force,        N      175.959           0.000

Speed,        m/s      4.937           0.000

Power,        W      552.192           0.000

TripDistance, m     5800.838

TripEnergy,   MJ       0.555

Data points        652

Calibration values:

gain             offset

Speed        -0.052           6.460

Force        -4.878         687.805

Volts         0.055           0.000

Wheel diam    0.088

# 15:11:47 14/6/2003 Barry Dunstan 7.25" Dolgoch 0-4-0

Important note: Due to a mistake in entering the dynamometer car wheel diameter, the figures for distance, speed, power and energy must all be reduced by 15%.

Summary values:

Max       Min

Force,        N      231.952           0.000

Speed,        m/s      5.670           0.000

Power,        W      768.528           0.000

TripDistance, m     6574.876

TripEnergy,   MJ       0.417

Data points        739

Calibration values:

gain             offset

Speed        -0.052           6.460

Force        -4.878         687.805

Volts         0.055           0.000

Wheel diam    0.088

# 12:14:27 14/6/2003 Bill Coles 7.25" Jens 0-4-0

Important note: Due to a mistake in entering the dynamometer car wheel diameter, the figures for distance, speed, power and energy must all be reduced by 15%.

Summary values:

Max       Min

Force,        N      105.017           0.000

Speed,        m/s      4.442           0.000

Power,        W      303.648           0.000

TripDistance, m     5800.838

TripEnergy,   MJ       0.220

Data points        652

Calibration values:

gain             offset

Speed        -0.052           6.460

Force        -4.878         687.805

Volts         0.055           0.000

Wheel diam    0.088

# 10:47:11 14/6/2003 Bob Nash 7.25" Koppel 0-6-0 Driven Glen Templeton

Important note: Due to a mistake in entering the dynamometer car wheel diameter, the figures for distance, speed, power and energy must all be reduced by 15%.

Summary values:

Max       Min

Force,        N      233.262          -0.959

Speed,        m/s      4.647           0.000

Power,        W      725.898          -3.275

TripDistance, m     5800.838

TripEnergy,   MJ       0.475

Data points        652

Calibration values:

gain             offset

Speed        -0.052           6.460

Force        -4.878         687.805

Volts         0.055           0.000

Wheel diam    0.088

# 12:58:47 14/6/2003 Bob Smith 5" SAR 702 2-8-2

Important note: Due to a mistake in entering the dynamometer car wheel diameter, the figures for distance, speed, power and energy must all be reduced by 15%.

Summary values:

Max       Min

Force,        N      134.494           0.000

Speed,        m/s      4.580           0.000

Power,        W      381.876           0.000

TripDistance, m     5418.267

TripEnergy,   MJ       0.243

Data points        609

Calibration values:

gain             offset

Speed        -0.052           6.460

Force        -4.878         687.805

Volts         0.055           0.000

Wheel diam    0.088

# 15:55:57 14/6/2003 Bryan Homann 5" Gresley 2-8-0

Important note: Due to a mistake in entering the dynamometer car wheel diameter, the figures for distance, speed, power and energy must all be reduced by 15%.

Summary values:

Max       Min

Force,        N      179.443           0.000

Speed,        m/s      5.788           0.000

Power,        W      635.642           0.000

TripDistance, m     6574.876

TripEnergy,   MJ       0.402

Data points        739

Calibration values:

gain             offset

Speed        -0.052           6.460

Force        -4.878         687.805

Volts         0.055           0.000

Wheel diam    0.088

# 11:29:30 14/6/2003 Graham Driscoll 7.25" 4 truck Shay

Important note: Due to a mistake in entering the dynamometer car wheel diameter, the figures for distance, speed, power and energy must all be reduced by 15%.

Summary values:

Max       Min

Force,        N      127.761           0.000

Speed,        m/s      4.886           0.000

Power,        W      422.175           0.000

TripDistance, m     6192.305

TripEnergy,   MJ       0.330

Data points        696

Calibration values:

gain             offset

Speed        -0.052           6.460

Force        -4.878         687.805

Volts         0.055           0.000

Wheel diam    0.088

# 14:29:50 14/6/2003 Ian Thomas 7.25" Freelance 4-6-2

Important note: Due to a mistake in entering the dynamometer car wheel diameter, the figures for distance, speed, power and energy must all be reduced by 15%.

Summary values:

Max       Min

Force,        N      237.608           0.000

Speed,        m/s      4.961           0.000

Power,        W      748.701           0.000

TripDistance, m     6797.301

TripEnergy,   MJ       0.523

Data points        764

Calibration values:

gain             offset

Speed        -0.052           6.460

Force        -4.878         687.805

Volts         0.055           0.000

Wheel diam    0.088

# 13:46:6 14/6/2003 John Beal 5" Schenectady  0-6-0T

Important note: Due to a mistake in entering the dynamometer car wheel diameter, the figures for distance, speed, power and energy must all be reduced by 15%.

Summary values:

Max       Min

Force,        N       93.647           0.000

Speed,        m/s      4.019           0.000

Power,        W      175.525           0.000

TripDistance, m     4252.761

TripEnergy,   MJ       0.137

Data points        478

Calibration values:

gain             offset

Speed        -0.052           6.460

Force        -4.878         687.805

Volts         0.055           0.000

Wheel diam    0.088