- SASMEE Locomotive Efficiency Trials Day 2004

(Abridged article for SASMEE Bulletin)

By

- Allan Wallace

14/6/2004

At this time last year we ran the first SASMEE Locomotive Efficiency Trials day.  The event was repeated this year and Saturday 12 June 2004 became second SASMEE Locomotive Efficiency Trials Day.  Many of the same engines and drivers entered, with everyone expecting to improve on the previous result.

We used the same dynamometer car as before. Externally, it is a close 5” gauge replica of the prototype in its early livery.  Internally, it is very modern.  The front drawbar is linked to an electronic load cell, and the wheels drive a DC tachometer and an optical revolution counter.  A box of electronics conditions the signals, converts them to digital format, and transmits them to a laptop computer carried on a conventional passenger truck behind the car.

The laptop computer displays a scrolling “paper” chart, like the original instrument table, but it reads data every second and stores the information on disk for subsequent display or printing.

A full technical description of the dynamometer car can be downloaded from my website, at www.avocetconsulting.com.au/modeleng.  It was a joint project by John Lyas and me.   You can also download the graphical data from all the runs done during the trials day.

- Figure 1 is a typical display.

Figure 1

We used the outer (7.25”/5” loop for al the tests.   Locomotives towed the dynamometer car and a train for about 30 minutes. Coal was taken from a weighed batch and the firebox refilled to its starting depth at the end of the run.  Only the load from the dynamometer car back was included in the work done tally.

The grades around the circuit are not severe, but generally the drawbar pull becomes a push at some places when the locomotive slows down and the train pushes the engine.  The software was set to ignore negative drawbar force so that locomotive braking did not reduce the work tally.

To calculate the thermal efficiency, the useful work done at the drawbar (measured by the dynamometer car, in Joules) was divided by the energy released by the burnt coal.

- Figure 2 shows the relative thermal efficiencies in rank order.

Figure 2

It must be stated that quoting the figures to three significant figures is not justified, given the approximation involved in measuring the coal consumed.  The differences between the first three engines is much smaller than the measurement accuracy and they should be ranked as equal first.

Efficiency is the ratio of output/input, so it is a valid index for comparing locomotives of any scale.  Figure 3 breaks the efficiency into its two components for each engine, where drawbar power is the Output, and coal rate is a measure of Input.

Figure 3

Figure 4

In figure 4 the coal burnt (at a calorific value of 30 MJ/kg), and the work done, have both been divided by the running time to express them as watts, and kg/hr, which allow direct comparison.   The power of combustion in the small fireboxes is noteworthy, reaching around 30 kW.  This assumes that all the coal is burnt and does not allow for losses into the ashpan.

Compared to last year, the three top engines were working harder, and have returned better efficiency.  There would be an optimum power output where the efficiency is a maximum for any engine.  One of the challenges for drivers is to choose a load near to this optimum

We felt that the biggest shortcoming in the procedure was the measurement of coal consumed.  Even with the best intentions, one cannot be sure that the level of fuel in the firebox is the same before and after the run.

We took the opportunity to test a petrol-electric locomotive built and driven by Graham Brown.  The statistics for this engine were:

Average speed 		12.5 kph

Average drawbar power 		287 Watts

Test  time and distance		29.0 minutes, 6038 m

Mass of petrol used		607 g

Combustion rate (44.2 MJ/kg)	15419 watts

Thermal efficiency		1.86%

Graham’s motive power is a Victa two-stroke single cylinder engine with a displacement of 160 ml, driving two Bosch alternators which in turn drive four  permanent-magnet traction motors.

Curiously, last year’s winner (Speedy) returned only 1.0%, compared to 1.6% before, despite being driven under almost identical conditions.  This might be a reflection of the poor accuracy of the coal measurement, but there are other factors too.  Finer coal was used this year (more was lost to the ashpan) and no doubt the seals are leaking more, as she has not been serviced since the previous run.

As mentioned above, there was no significant difference between the first three engines, but if we accept the lottery of the third significant figure, then Dolgoch is the winner.

Dolgoch was originally built by Frank Lee, and has 2.187 bore x 3.5” stroke cylinders.  The pistons are fitted with cast iron rings.   Stephenson’s valve gear is used, with slide valves.  The driving wheel diameter is 7.25” and the axles run in bronze journal bearings.

Boiler feed is catered for by an axle pump, a donkey pump and an injector.

The copper boiler is 7.25” diameter with 18 firetubes and two stainless steel radiant superheaters.  It has a stainless steel arch and the grate has 20% open area with 3/32” gaps.

In her dynamometer data, she recorded a maximum drawbar pull of 460 Newtons and a peak power output of 670 watts.  It is interesting to see that the efficiency last year was 0.8% at an average power of 190 watts, compared to this year’s 1.7% at 290 watts.

Thanks to the committee, the helpers and the drivers for helping make this an enjoyable and instructive day.